### A Narragansett Bay Estuary Program Report

# Report on the Analysis of True Color Aerial Photographs to Map Submerged Aquatic Vegetation, Coastal Wetlands, Deepwater Habitats and Coastal Features in

# Southern Rhode Island and Southeastern Connecticut

Narragansett Bay Estuary Program

Prepared by

Irene Huber

#### Natural Resources Assessment Group

### Department of Plant and Soil Sciences

University of Massachusetts

Amherst, Massachusetts

June 2003

#NBEP – 04-122

## Funded by

## U.S. Environmental Protection Agency

## Narragansett Bay Estuary Program

## R.I. Department of Environmental Management

R.I. Oil Spill Prevention, Administration and Response Fund

A.  INTRODUCTION

The University of Massachusetts, Natural Resources Assessment Group (NRAG) contracted in 1999 with Rhode Island Department of Environmental Management (RIDEM), and with the Narragansett Bay Estuary Program (NBEP) to provide original air photo interpretation of the submerged aquatic vegetation (SAV) primarily eelgrass (Zostera marina), coastal wetlands, shoreline, and selected coastal features in southern Rhode Island, including Block Island, and southeastern Connecticut.  The project was undertaken in cooperation with the U.S. Fish & Wildlife Service, Northeast Region (FWS).  Under cooperation, work included inventory of freshwater wetlands on the quadrangle covered for the coastal zone.

The University of Rhode Island, Environmental Data Center (URI/EDC) was the GIS contractor for the data, which was delivered as a scanned product.  Scanning of data was conducted by the FWS.

For purposes of this report, areas inventoried other than SAV are referred to as “coastal habitats.” The SAV, coastal wetlands and selected coastal features were classified according to the U.S. Fish and Wildlife Service’s Classification of Wetlands and Deepwater Habitats of the United States (Cowardin et al. 1979; reprinted 1992).

Z. marina is recognized as a vital resource for food chain support and as a positive indicator species of estuarine productivity and function.  An inventory of Z. marina beds provides resource managers with means to assess the extent and locations of this resource.  Additionally, the inventory provides managers with a reasonably accurate acreage and location of present coastal habitats by type.  The data base can be further developed with specialized analyses such as trends analysis and mapping of potential coastal wetland restoration sites.  The eelgrass data base can be modified to accommodate restoration efforts, loss, and shifts in the bed distribution.

The report describes methods employed by NRAG to produce the inventory and details field findings.   Appendix A is a plant list for the project area derived from field work conducted by NRAG to verify interpretation.  Appendix B contains field data sheets for 117 field sites documented with the project.  Additionally, results of the field work and photo interpretation are summarized, and acreage statistics by habitat type are provided.

No freshwater wetlands or freshwater deepwater habitats are mapped with this project. However, freshwater wetlands were field documented where establishing the limits of estuarine habitats; (oligohaline conditions were required).

B.  PROJECT AREA

The South Shore project area covers the extreme southeastern Connecticut and southern Rhode Island coastline, including Block Island.  The mainland area extends from Stonington, Connecticut, near the mouth of the Pawcatuck River, easterly to Point Judith, and includes the coastal zone from Sakonnet Point easterly to the state line of Massachusetts.  The mapping of SAV and coastal habitats was performed for the following municipalities: Stonington (partial coverage), Connecticut, and Westerly, Charlestown, South Kingstown, Narragansett (partial coverage), Little Compton, and New Shoreham (Block Island).

Landward limits of the project area were defined by the limits of brackish classification (Cowardin et al, 1979).  See Section C3.

The South Shore project area ties to a previous inventory of SAV and coastal habitats for Narragansett Bay, produced by NRAG for NBEP and Save The Bay, Inc. (STB).  (See Report on the Analysis of True color Aerial Photographs to Map Submerged Aquatic vegetation and Coastal Resource Areas in Narragansett Bay Tidal Waters and Near Shore Areas, Rhode Island and Massachusetts.)

The project area is found on the following U.S.G.S. topographic quadrangle maps:  Mystic, Watch Hill, Quonochontaug, Carolina, Kingston, Narragansett Pier, Sakonnet Point and Block Island.

C.  METHODS AND MATERIALS

- Aerial Photograph Acquisition

Aerial photographs were ordered by NBEP with specifications from NOAA Coastal Change Analysis Program (C-CAP): Guidance for Regional Implementation (Dobson et. al, 1995) for obtaining optimal SAV visibility.  Two overflights were used:  1:12,000 scale (1:12K) true color transparencies for SAV photo interpretation, and 1:40,000 scale (1:40K) true color transparencies for photo interpreting estuarine and marine habitats.  The 1:12K overflight was obtained to maximize detection of SAV, primarily Z.marina, under conditions approaching peak biomass, low haze, low wind speed, minimal to no cloud cover, low tide and low turbidity.  The 1:40K overflight used for coastal habitat mapping allowed reasonably accurate mapping with respect to production costs, efficiency of transfer, and targeted minimum mapping units for GIS map production and display.

Research personnel from the University of Rhode Island Graduate School of Oceanography (URI-GSO) provided information to the NBEP on the peak biomass of eelgrass and water clarity in the coastal ponds.  Their information assisted with instruction to the air photo contractors regarding optimal time of overflight for eelgrass detection.

Both overflights were taken on June 5, 1999 by the James W. Sewell Company, Old town, Maine.  Photography was reviewed by NRAG and determined suitable quality for purposes of the project.

- Data Preparation

NRAG prepared aerial photos for photo delineation in the following manner:

  - Each model was mounted with a Grafix Wet Media Dura-Lar .004 gauge 9 inch by 9 inch Mylar, affixed with drafting tape at each corner.

  - Each work area Mylar was pin-registered to each photo model at four corners.

  - Identifying notations in permanent black ink were made on each work area Mylar (photo number, adjoining photo number and edge lines).

  - Labels identifying photo number, date, scale and project were affixed on the upper edge of the photos.

  - Work area photo edge lines allowed for distortion at outer edges of the photos while accommodating tie area overlap.

  - Photography was indexed and organized into separate folders by flight lines and quadrangles.

- Determining Project Limits and System Breaks

Landward limits of the project area were determined both in the field and during photo interpretation as the limits of estuarine brackish vegetation or brackish deepwater habitat, with respect to the following definitions in Cowardin et al., 1979:

The estuarine system extends (1) upstream and landward to where ocean-derived salts measure less than 0.5 parts per thousand during the period of average annual flow;  (2) an imaginary line closing the mouth of a river, bay or sound; and (3) to the seaward limits of wetland emergents, shrubs or trees where they are not included in (2).

Seaward project limits were established by defining the mouth of Little Narragansett Bay in Watch Hill, the openings of breachways or inlets of coastal lagoons (Winnapaug, Quonochontaug, Ninigret, Green Hill, Point Judith and Great Salt Ponds).  Where coastal lagoons are not breached, estuarine conditions were determined from the Rhode Island Coastal Resources Management Program, As Amended and/or from personal references. The system breaks between the Estuarine and Marine environments were established with respect to Cowardin et al., 1979:

The Marine System extends from the outer edge of the continental shelf shoreward to one of three lines:  (1) the landward limit of tidal inundation (extreme high water of spring tides), including the splash zone from breaking waves; (2) the seaward limit of wetland emergents, trees, or shrubs; or (3) the seaward limits of the Estuarine System, where this limits is determined by factors other than vegetation.

It is noted that freshwater wetlands and deepwater habitats, including those classified in Cowardin et al. (1979) as “tidally-influenced” are not included in the inventory.  Users are referred to Cowardin for further definition of the tidally-influenced freshwater system.  Some wetland areas observed during photointerpretation appear to be former estuarine wetlands, altered by restriction, ditching or other activities which changed classification from estuarine to tidally-influenced freshwater, and are therefore not mapped with this inventory.  Such areas, however, may be subject to future analysis as potential estuarine wetland restoration sites.

- Minimum Mapping Units

Minimum polygonal mapping units (MMU) targeted for this project are 0.5 acre for Z. marina beds and .25 acre for isolated polygonal coastal wetlands and deepwater habitats.  The internal targeted MMU (wetland types within other wetland types) was contracted to be 3.0-to-5.0 acres; however, smaller internal polygons of around 1.0 acre in size were mapped where types of particular ecological significance are found (e.g., pools within high marshes).  Inclusion of small significant habitat types was based on photo quality, photointerpreter judgment and field site information.

It is noted that excessive internal mapping can create difficulties for project production costs and GIS map display.  The level of mapping detail is dictated by the scope of the project, photo quality, the scale of photography used and photointerpreter judgment.  The MMU for linear estuarine and marine habitats was pen width or approximately 35 feet on the ground using 1:40K scale photography.  Collateral use of the 1:12K photography for verification of fringe marsh and beaches resulted in linear features slightly less than pen width, except where shadowed on either source photography.  Therefore, an estimated average width of 27.5 feet was used to calculate acreage statistics.  The linear foot (LF) MMU for line segments was 1/8 inch or about 250 LF at 1:24K base map scale.  In contrast to the bay inventory all aquatic beds were mapped as polygonal data for the South Shore provided they were photointerpretable.

5. Field Work

Prior to photointerpretation, NRAG staff previewed aerial photos, selected field sites, and noted them on USGS 1:24K topographic maps for in-field orientation.

Field data was collected for two purposes:

1) to discriminate photosignatures unique to various habitat types as workable with the type and scale of source photography, and

2) to provide an ecological profile of habitat types representative of the project area.

Site selection criteria included the following:

a) areas representative of project area ecology; b) areas disturbed and potentially requiring NWI modifiers in classification; c) areas used to establish system and sub-system classification breaks; d) areas affected by haze, shadow, emulsion or other photography quality concerns; e) areas accessible with respect to trespassing and time constraints.

The NBEP coordinated the SAV field work in the coastal ponds, Little Narragansett Bay, and Great Salt Pond in Block Island.  The RIDEM Office of Water Resources-Shellfish Program, Division of Enforcement, Division of Fish and Wildlife, Narragansett Bay Estuary Program, New Shoreham Harbor Patrol, and Brown University provided personnel and boats to the NRAG to perform fieldwork in August and September 1999.  A total of 80 SAV field sites were inspected and documented by boat observation and sampling.  No dives were undertaken.

A total of 37 estuarine and marine habitat sites were inspected and documented by vehicle and by foot, primarily coastal wetlands, lagoons, and beach habitats.

Field data sheets were developed by NRAG specifically for the project.

- Aerial Photointerpretation and Quality Control

Photodelineation utilized Cartographic Engineering mirrored stereoscopes and rapidographs at 4x0 line weight with permanent black India ink.

For mapping SAV, use of Cowardin et al. (1979) permits description of the beds to life form and water regime (see Tables 2 and 3).  Z. marina was differentiated from other SAV species by the subclass “3,” rooted vascular; however, there was one field verified bed of Ruppia which is classified the same as Zostera (Ninigret Pond).

Map classification and delineation techniques for the SAV, coastal wetlands and deepwater habitats were based on Cowardin et al. (1979) and the accompanying Photointerpretation Conventions for the National Wetlands Inventory (National Wetlands Inventory, 1995).  The coastal wetland data presented with this project as a mapping product is not intended to substitute for on-site determinations or delineations in permitting.  The mapped delineations of coastal wetlands are not to be transferred or represented for regulatory purposes.

Coastal features included in the inventory are dunes and coastal banks.  Photointerpretation of these features was as best determined on the 1:40K photography and with reference to definitions in the Rhode Island Coastal Resources Management Program, As Amended, (RICRMP) Sections 210.  These features are mapped using photointerpretation techniques, and are not to be transferred or represented for regulatory purpose, and cannot substitute for on-site regulatory determinations or delineations in state or other permitting. Additionally, mapping of coastal bank and dune polygons assists with display of the data, in that these features are distinguished from upland islands and inclusions in the coastal zone.

The shoreline delineation is calculated from the landward limits of mapped linework (polygon and linear).  The shoreline representation is a product of a mapping effort, and is not intended for regulatory purpose.

For quality control of the photointerpretation, each completed annotated mylar was examined by a photointerpreter other than the one producing the original photodelineations and classifications.  Corrections were made as needed to maintain accuracy and consistency throughout the map product.

Photointerpretation and quality control progressed in a south to north, quad by quad basis to maintain delivery for rectification and digitizing.

- Transfer, Rectification and Base Map Preparation:

Transfer of SAV data from the 1:12K to the 1:40K photography took place using a Bausch & Lomb Stereo Integrated System (SIS) or by use of a Zoom Transfer Scope (ZTS).

Data on the 1:40K photography were rectified using a Bausch & Lomb Zoom transfer Scope (ZTS).  Data were transferred from the aerial photograph overlays to USGS 1:24K stable base mylars affixed with registered frosted mylar overlays.  Frosted mylars containing the rectified data were fist quality controlled by NRAG staff, rectified data was then scanned by FWS and sent to URI/EDC.  Quality control of rectified data addressed alignment, labeling and linework completions prior to delivery for digitizing.

The NBEP coordinated the review of the draft Z. marina GIS coverage by RIDEM Division of Fish and Wildlife and URI Graduate School of Oceanography personnel.  The NRAG staff responded to comments and adjusted delineations as needed.

- Habitat Type Classification

Within estuarine and marine habitats (or “systems”), subtidal and intertidal sybsystems were applied according to the following definitions from Cowardin et al. (1979):

Subtidal (1). – The substrate is continuously submerged.

Intertidal (2). – The substrate is exposed and flooded by tides; includes the associated splash zone.

Tables 2 and 3 are provided with reference to Cowardin et al. (1979) defining particular habitat types inventoried with this project.  Table 2 describes tidal water regimes and Table 3 summarizes the classification of habitat types inventoried.

Classification includes mixes of subclasses where subordinate cover is at least 30%.

Table 2. Tidal Water Regimes and Special Modifiers for Habitat Types in the Narragansett Bay Project Area

Tidal Water Regimes

Subtidal (L.)  The substrate is permanently flooded with tidal water.

Irregularly Exposed (M).  The land surface is exposed by tides less often than daily

Regularly Flooded (N).  Tidal water alternately floods and exposes the land surface at least once daily.

Irregularly Flooded (P). Tidal water floods the land surface less often than daily.

Special Modifiers

Excavated (x):  Lies within a basin or channel excavated by man.

Impounded (h):  Created or modified by a barrier or dam which purposefully or unintentionally obstructs the outflow of water; includes man-made dams and beaver dams.

Diked (h):  Created or modified by a man-made barrier of dike designed to obstruct the inflow of water.

Ditched/Partly-Drained (d):  The water level has been artificially lowered, but the area is still classified as wetland because soil moisture is sufficient to support hydrophytes.

Artificial (r): Refers to substrates classified as Rock Bottom, Unconsolidated Bottom, Rocky Shore and Unconsolidated Shore that were emplaced by man, using either natural materials such as dredge spoil or synthetic materials such as …concrete.  Jetties and breakwaters are examples of Artificial Rocky Shores.

Oligohaline (6):  Term to characterize water with salinity of 0.5 to 5.0 parts per thousand, due to ocean-derived salts.

Table 3.  NWI Classification Codes and Descriptions, South Shore Project Area.

| NWI Code & Modifiers | Cowardin et al. (1979) Description | Common Description | Examples of Vegetation or Cover |
| --- | --- | --- | --- |
| EIUB, MIUB (L, Lh, Lx) | Estuarine, Marine Subtidal, Unconsolidated Bottom | Estuarine, or Marine Open Water | Open water (includes open ocean, lagoons & tidal creeks) |
| E1AB3L; M1AB3L | Estuarine or Marine, Subtidal, Rooted Vascular Aquatic Bed | Eelgrass Bed | Zostera marina |
| E1AB1L | Estuarine, Subtidal Algal, Aquatic Bed | Algal Beds | Ulva lactuca, Fucus spp., Chondrus crispus, Enteromorpha sp. |
| E1UB4L | Estuarine, Subtidal, Unconsolidated Bottom, Organic | Pools | Ruppia sp. or other algae |
| E2US4(N,M) | Estuarine, Intertidal Unconsolidated Bottom Organic | Pannes | Scant Salicornia sp. |
| E2US(1,2,3,)(M,N) | Estuarine, Intertidal Unconsolidated Shores | Tide Flats | Cobble, gravel, sand or mud; patches of algae |
| E2RS(1,2)(N,P); M2RS(1,2)(N,P) | Estuarine or Marine, Intertidal Rocky Shores | Rocky Shores | Bedrock or rubble; patches of Fucus sp. |
| E2RF2N | Estuarine, Intertidal, Mollusc Reef | Oyster Bed | Crassostrea virginica |
| E2SB(2,3)(N) | Estuarine, Intertidal Streambed | Tidal Creek | Sand or mud |
| E2EM(1,5*)P | Estuarine, Intertidal Persistent Emergents, Irregularly Flooded | High Marsh | Spartina patens, Juncus gerardii, Distichlis spicata (*note: 5= Phragmites australis) |
| E2SS1P | Estuarine, Intertidal Scrub-Shrub, Broad-leaved Deciduous, Irregularly Flooded | High Marsh | Iva frutescens, Baccharis halimifolia |
| E2EM1N | Estuarine, Intertidal Persistent Emergents Regularly Flooded | Low Marsh | Spartina alterniflora |
| E2EM(1,5*)P6 | Estuarine, Intertidal Persistent Emergents Irregularly Flooded, Oligohaline | Brackish Marsh | Typha angustifolia, Spartina pectinata (*note: 5 = Phragmites australis) |

D. Results:  Descriptions and Acreages of Habitat Types

The acreages below were compiled from RIGIS for linear and polygonal data.  Table 4 is an acreage summary of habitat types inventoried with this project.  Linear acreages have been incorporated into the acreage summaries with the polygonal data.  Information on linear acreages as discrete tabulation may be obtained from URI/EDC.

Acreage of upland (islands and inclusions) was not reported with the summaries.  It is suggested these acreages may also be obtained from URI/EDC.

- Aquatic Beds

Various aquatic beds (SAV) were found dominated by one of the following species: eelgrass (Z. marina), Irish moss (Chondrus crispus), Deadman’s fingers (Codium fragile), barrel weed (Champia parvula), graceful red weed (Gracilaria sp.), rough tangleweed (Stilophora rhizodes), Sargassum (Sargassum filipendula),  widgeon grass (Ruppia maritim), knotted wrack (Ascophyllum nodosum), hollow green weeds (Enteromorpha spp.), rockweed (Fucus vesiculosus), and sea lettuce (Ulva lactuca).

Some common associations of SAV species included C. crispus/U. lactuca, C. fragile/Fucus sp., U. lactuca/brown algae and C. parvula/C. fragile.

Pure stands of Z. marina were found in about 30% of the sites inspected.  Remaining Z. marina beds inspected were mixed in composition, and included subordinate species such as Enteromorpha sp., C. crispus, Gracilaria sp., and C. fragile.

A total of 554.9 acres of Z. marina were inventoried.  The largest areas of eelgrass beds are in Ninigret Pond with 161.3 acres and Potter’s Pond with 122.3 acres.

By cover, Potter Pond has the highest eelgrass acreage to pond area.

### Table 4.   Eelgrass Acreage for Rhode Island South Shore Coastal Ponds

| ## Pond Name | NWI Code | Eelgrass Acres* | Pond Acres** | Percent Eelgrass |
| --- | --- | --- | --- | --- |
| Potter Pond | E1AB3L | 122.3 | 362.1 | 33.8 |
| Green Hill Pond | E1AB3L | 72.5 | 420.3 | 17.3 |
| Quonochontaug Pond | E1AB3L | 87.7 | 745.3 | 11.8 |
| Ninigret Pond | E1AB3L | 161.3 | 1,580.4 | 10.2 |
| Little Narragansett Bay | E1AB3L | 61.4 | 2,446.0 | 2.5 |
| Point Judith Pond | E1AB3L (dominating) E1AB1/3L (co-dominating) | 18.2 11.0 (total 29.2) | 1,548.2 | 1.9 |
| Trim’s Pond | E1AB3Lh | 13.7 | N/A | N/A |
| Great Salt Pond | E1AB3L M1AB3L | 3.4 3.4 (total 6.8) | N/A | N/A |
| Trustom Pond |  | 0 | 181.0 | 0 |
| Card Pond |  | 0 | 40.1 | 0 |
| Winnapaug Pond |  | 0 | 473.0 | 0 |
| Little Maschaug Pond |  | 0 | 11.7 | 0 |
| Maschaug Pond (B1) |  | 0 | 34.6 | 0 |
| TOTAL EELGRASS |  | 554.9 | ---- | ---- |

*Eelgrass acres reported with photo-analysis of 1999 source imagery.

** Pond acres from http://seagrant.gso.uri.edu/coasts/index.html

N/A = pond size not available.

- Dunes

A total of 244.5 acres of dune were inventoried with the project.  There may be additional acres of this habitat not mapped due to photography overexposure and low or shallow crests not detectable on the source photography.  Dunes are dynamic features and have likely changed in position and extent since the date of overflight.

- Open Water

A total of 112,964.7 acres of open water habitat was inventoried.  Estuarine open water accounted for 7,103.9 acres (6%). Marine open water amounted to 105,860.9 acres (93%).

- Tidal Flats and Pools

Estuarine tidal flats and pools totaled 1,738.4 acres.  Of this acreage, 1,639.6 acres were classified as tide flats and 98.8 acres were classified as pools.  Although no pannes of minimum detectable size were mapped, occurrence of these relatively small habitats is expected and would be inclusive with high salt marsh.  A more detailed inventory of pannes would require additional effort with higher scale photography.

- Beaches

A total of 856.1 acres of beaches were inventoried.  Approximately 26% (219.9 acres) were in the estuarine environment and 74% (636.2 acres) were marine habitats.  Of this acreage, regularly and irregularly-flooded estuarine sand material accounted for 198.8 acres and regularly and irregularly-flooded estuarine cobble material accounted for 21.1 acres.  Regularly and irregularly-flooded marine sand material accounted for 540.6 acres and regularly and irregularly-flooded marine cobble material accounted for 95.6 acres.

- Artificial Habitats

A total of 19.3 acres of rock jetties and groins were detectable as artificially-placed habitats.

- Rocky Shore

Rocky shores totaled 191.4 acres, with 8% or 15.6 acres in the estuarine environment and 92% or 175.8 acres in the marine environment.  Seaweeds such as Ascophyllum nodosum and Fucus vesiculosus commonly colonize the regularly-flooded zone of rocky shores.

- Reefs

Four acres of oyster reef (Crassostrea virginica) were inventoried in the estuarine system, 2 acres of which are interpreted as farmed (E2RF2Nr).

- Streambeds

There were 6.3 acres of estuarine mud or sand bottom streambed inventoried.

- Estuarine Emergent Low Salt Marsh

A total of 70.2 acres of estuarine low salt marsh (regularly-flooded) dominated by salt marsh cordgrass (Spartina alterniflora) was inventoried. One percent (.837 acres) is impounded and 4.4 acres (6.3%) are ditched.  Fringe acreage is included where detectable on source imagery.

- Estuarine Emergent High Salt Marsh

A total of 1324.6 acres of estuarine emergent high salt marsh (irregularly-flooded) was mapped.  Typical high marsh emergent species is represented by species such as salt meadow cordgrass (Spartina patens), black grass (Juncus gerardii), spike grass (Distichlis spicata), marsh orach (Atriplex patula), sea blite (Suaeda linearis), seaside arrow grass (Triglochin maritimum), annual salt marsh aster (Aster subulatus), perennial salt marsh aster (A. tenuifolius), seaside goldenrod (Solidago sempervirens) and sea lavender (Limonium nashii).  Of this acreage, 559.6 acres was ditched emergent high salt marsh.

- Phragmites Marsh

Common reed (Phragmites australis) dominated 7% (101 acres) of estuarine high salt marsh inventoried, and 53.1 acres (52.6%) of the P. australis-dominated high salt marsh was ditched.

- Estuarine Scrub-Shrub and Brackish Shrub Wetland

Estuarine high salt marsh (irregularly-flooded) dominated by shrubs accounted for 113.6 acres.  Of this, 40 % (45.7 acres) was ditched.  Species representative of this habitat type are high tide bush (Iva frutescens) and groundsel tree (Baccharis halimifolia).

- Estuarine Brackish Marshes

Brackish estuarine emergent marsh totaled 116.5 acres.  Typical brackish marsh communites were found in the Kingston and Narragansett Pier quads.  Three of the field sites were dominated by the narrow-leaved cattail (Typha angustifolia) and one was dominated by slough grass (Spartina pectinata).  Other species associated with the sites were common reed (Phragmites australis), wool grass (Scirpus cyperinus), swamp rose mallow (Hibiscus moscheutos), swamp rose (Rosa palustris), purple loosestrife (Lythrum salicaria), and curly dock (Rumex crispus).

- Phragmites Brackish Marsh

Common reed (Phragmites australis) in brackish conditions totaled 177.3 acres.

16.  Coastal Bank

A total of 84.6 acres of coastal bank was inventoried with the project.  Polygon-sized banks only were interpreted.  Additional acres are expected due to shadowing and size limitations when photo interpreting coastal banks.

- Upland

Within project area limits, the upland acreage was not provided.

Table 5.   Acreage Summary of Estuarine and Marine Habitats Inventoried for RI South Shore Project Area.

| Habitat Type | Acres |
| --- | --- |
| Eelgrass Beds | 570.3 |
| Dunes | 244.5 |
| Open Water | 112,964.7 |
| Pools | 116.9 |
| Tidal Flats | 1,621.5 |
| Beaches | 856.1 |
| Artificial Jetties & Breakwaters | 19.3 |
| Rocky Shores | 191.4 |
| Oyster Reefs | 4.4 |
| Stream Beds | 6.3 |
| High Salt Marsh | 1,425.6 |
| High Scrub-Shrub Marsh and Brackish Scrub-Shrub Marsh | 113.6 |
| Low Salt Marsh | 70.2 |
| Brackish Marsh | 293.8 |
| Coastal Bank | 84.6 |
| GRAND TOTAL | 118,583.2 |

E.  Photointerpretation Problems and Resolution

- Detection of Z. marina Beds

Positive depth change appears to affect photodelineation of the outer limits of eelgrass beds independent of turbidity conditions.

It is suggested  small Z. marina beds verified by field personnel may be added as points to the data base using locations aids such as the Global Positioning System (GPS).  Area measurements of selected Z. marina polygons may be obtained by dives or other methods (e.g. hydroacoustic detection) to refine delineations at outer limits.  Bathymetry data might be a helpful correlation with as-mapped Z. marina polygons to estimate outer portions of beds within limits of light penetration required for survival.

It is noted that data with this inventory are based on 1999 conditions.  Any eelgrass transplants since the time of overflight may be added to the database.

- Shadowing

Shadows from relatively tall upland features such as conifer tree stands, banks and tall buildings were present on portions of the 1:12K and the 1:40K overflights.  This condition is not expected to have significantly affected photointerpretation within established minimum map units, and is a common limitation in photointerpretation.  Areas shadowed from the upland would be narrow habitats along the shore and perhaps upper limits of high marshes.  Problems with shadowing were reduced by use of collateral data and by interpolating between non-shadowed areas.

- Haze and Cloud Cover

The 1:40K overflight was hazy on portions, but overall quality was good.  Although the 1:12K-scale photography was considered excellent quality, there were still limitations on the detection of the outer edge of the eelgrass beds with increasing depths.  This results from reduced reflectance contrast between the vegetation and deepening open water.

- Minimum Mapping Units

Detection of small eelgrass beds (that is, .25 to about 3.0 acres) may be restricted and/or lack of reflectance contrast.  We suggest supplementing the data base with point locations of Z. marina confirmed by field personnel (See Section E.1.).  Inventory of estuarine and marine habitats was based on photointerpretation of the 1:40K imagery.  We found this imagery successful to meet the minimum mapping units.  Users may be interested in additional detail on wetland plant communities for example, small (less than 1.0 acre) internal communities of P. australis or I. frutescens.  Large scale (eg. 1:12K) photointerpretation would enable this, with caution that proper rectification methods may be costly.

5)  Errors in Data

A draft map review was undertaken after the transfer and digitizing to verify addition of smaller habitat areas, verify classification, and to proof delineations and labels.

F. Summary

A total of 118,583.2 acres of coastal wetlands, deepwater habitats and coastal features were inventoried with this project.  Of the total, 570.3 acres of eelgrass; 1,903 acres of coastal marshes; 117 acres of tide pools; 1,628 acres of tidal flats and streambeds; 856 acres of beaches; 191 acres of rocky shore; 244 acres of dune; 85 acres of coastal bank; 4 acres of oyster reef;  and 19 acres of jetties ad breakwaters were inventoried.

Using the 1:12K imagery provided for this project, a reasonably conclusive photosignature unique to Z. marina was established for interpretation.  Various field-to-photo signature correlations from other kinds of algal beds further defined photosignature characteristics of Z. marina.  Field verification of eelgrass by boat sampling and observation was conducted, supported by various collateral information for sites not accessed during field work.

Beds were either purely Z. marina or dominated by the species.

Numerous small sized Z. marina beds are not likely mapped with this inventory.  This may be attributed to lack of reflectance contrast in the ponds.  It is suggested that point locations of small Z. marina beds be added to the data base using field personnel and location aids such as GPS.

Outer limits of Z. marina beds were not able to be accurately represented.  As beds drop off with depth, reflectance contrast between the vegetation and the open water is reduced.  Turbidity may further confound accurate interpretation of outer boundaries.

Z. marina was inventoried in estuarine (about 553 acres) and in marine (18 acres) environments. Highest areas and densities of eelgrass occur in Ninigret and Potter Ponds.

The locations, extent, and acreages of estuarine and marine habitats may be used to perform analysis of potential wetland restoration sites; for example, tidally-restricted wetlands, filled wetlands, and wetlands colonized by Phragmites australis.  A trends analysis may be helpful to determine acreages and possible causes of coastal habitat loss and degradation over selected time intervals.

Increased map detail on the extent and locations of Phragmites australis, or other plant communities of special interest, is possible using larger scale imagery and transferring the information to the data base, as was performed for the Narragansett Bay Coastal Wetland Restoration Project.  Data on tidally restricted marshes may locate opportunities for salt marsh restoration.

ACKNOWLEDGMENTS

We acknowledge Helen Cottrell, who served as project coordinator for RIDEM Narragansett Bay Estuary Program.  Additionally, Ms. Cottrell assisted with draft review of this report.

NRAG gratefully thanks the staff of RIDEM Office of Water Resources Shellfish Program and Division of Enforcement for assisting biologists with boat transport for aquatic bed sampling in the field.  Individuals are John Speaker, Tom Iarossi, Frances Ethier and Ed Cabral.  Additionally, verbal information provided by Art Ganz and John Gingerella of RIDEM Division of Fish and Wildlife is acknowledged.  The field efforts of Chris Powell RIDEM Fish and Wildlife, who conducted aquatic bed verification on Block Island and in Sakonnet, are greatly appreciated, as inclement weather impacted certain areas of our field work.  Larry Constantine, New Shoreham Harbor Master, also provided boat transport to conduct field work in the Great Salt Pond.  Leslie Katz of Brown University assisted with SAV field verification on Block Island.

We thank Ralph Tiner, U.S. Fish & Wildlife Service’s Regional Wetlands Coordinator, for acting as cooperator and consultant on this project.  Gabriel D’Alessio of the U.S. Fish & Wildlife Service’s National Wetlands Inventory, Regional Office scanned and cleaned rectified raw data for the project.

Aimee Mandeville of URI Environmental Data Center conducted GIS work.

Peter Veneman of UMASS acted as principal investigator on this project.  We thank Mary Johnson, UMASS Department of Plant & Soil Sciences Graduate program, for assisting with Phragmites detection, report documentation and plant lists.  We thank Meaghan Shaffer of NRAG for assisting with documentation and organizing plant community descriptions for this report.  Todd Nuerminger and Irene Huber performed field work, original air photointerpretation, quality control and documentation for this project.

The following papers provided collateral information on location of eelgrass beds:

Granger S., S. Nixon, and B.Allen. 2000. Field Observations of Seagrass Coverage and Water Quality in Ninigret, Green Hill, Quonochontaug and Winnapaug Ponds. Final report submitted to U.S. Army Corps of Engineers. URI-School of Oceanography, Narragansett, R.I.

Field Observations in Block Island and Little Compton by Chris Powell, RIDEM Division of Fish and Wildlife, Marine Fisheries Section, Jamestown, R.I., 2000.

Field Observations in Little Narragansett Bay by Robinson Fulweiler, URI Graduate School of Oceanography, Narragansett, R.I., 2001.

Funding for this project was provided by the R.I. Oil Spill Prevention, Administration and Response Fund, U.S. Environmental Protection Agency, Narragansett Bay Estuary Program, and R.I. Department of Environmental Management.

REFERENCES

Cowardin, L.M., V. Carter, F.C. Golet, and T. LaRoe. 1979.  (Reprinted 1992).  Classification of Wetlands and Deepwater Habitats of the United States.  U.S. Fish & Wildlife Service, Washington, DC.  FWS/OBS-79/31.  103 pp.

Dobson, J.E., E.A. Bright, R.L. Ferguson, D.W. Field, L.L. Wood, K.D. Haddad, H. Iredale III, J.R. Jensen, V.V. Klemas, R. J. Orth, and J.P. Thomas.  1995.  NOAA Coastal Change Analysis Program (C-CAP):  Guidance for Regional Implementation.  U.S. Department of Commerce, Seattle, WA.  NOAA Tech. Dept. NMFS 123. 59 pp.

National Wetland Inventory.  1990.  Photointerpretation Conventions for the National Wetland Inventory.  U.S. Fish & Wildlife Service, St. Petersburg, FL 45 pp.  plus appendices.

Orth, R. J., J.F. Nowak, G.F. Anderson and J.R. Whiting.  1994.  Distribution of Submerged Aquatic Vegetation in the Chesapeake Bay and Tributaries and Chincoteague Bay – 1993.  Virginia Institute of Marine Science, Gloucester Point, VA. 262 pp.

Reed, P.B., Jr. 1988.  National List of Plant Species that Occur in Wetlands:  Northeast (Region 1).  U.S. Fish & Wildlife Service.  Biol. Rept. 88(26.1).  111 pp.

Tiner, R.W., Jr. 1987.  A Field Guide to the Coastal Wetland Plants of the Northeastern United States.  The University of Massachusetts Press, Amherst, MA.  272 pp.

Villalard-Bohnsack, M.  1995.  Illustrated Key to the Seaweeds of New England.  The Rhode Island Natural History Survey, University of Rhode Island, Kingston, RI. 139 pp.

# Appendix A

# Plant Species Observed at Field Sites

# Plant Species Observed at Field Sites

South Shore Project Area, Rhode Island

August 1999, September 1999, November 1999

May 2000, August 2000

- Estuarine Wetlands:

## Scientific Name	Common Name

### Agalinis maritima	Saltmarsh False-Foxglove

### Aster subulatus	Annual Saltmarsh Aster

### Atriplex petula	Halberd-Leaf Saltbush

### Baccharis halimifolia	Sea Myrtle

### Cyperus filicinus	Slender Flatsedge

### Distichilis spicata	Seashore Saltgrass

### Hibiscus moscheutos	Swamp Rose Mallow

### Iva frutescens 	Marsh Elder

### Juncus gerardii					Saltmeadow Rush

### Limonium carolinianum				Carolina Sea-Lavender

### Limonium nashii					Northern Sea-Lavender

### Lythrum salicaria					Purple Loosestrife

### Panicum virgatum					Switchgrass

### Phragmites australis					Common Reed

### Plantago maritima	Seaside Plantain

### Potentilla anserina	Silverweed

### Puccinellia maritima	Seaside Alkali Grass

### Rosa palustris	Swamp Rose

### Rumex crispus	Curly Dock

### Salicornia sp.	Glasswort

### Salicornia europaea	Slender Grasswort

### Scirpus americanus	Olney’s Bulrush

### Scirpus cyperinus	Wool Grass

### Scirpus pungens	Three-Square Bulrush

### Solidago sempervirens	Seaside Goldenrod

### Spartina alterniflora	Saltmarsh Cordgrass

### Spartina patens 	Saltmeadow Cordgras

### Spartina pectinata	Slough Grass

Toxicondendron radicans	Poison Ivy

### Typha angustifolia					Narrow-leaved Cattail

- Submerged Aquatic Vegetation (SAV):

## Scientific Name	Common Name

### Ascophyllum nodosum	Knotted Wrack

### Codium fragile	Deadman’s Fingers

### Enteromorpha sp.	Hollow Green Weeds

### Fucus vesiculosus	Rockweed

### Gracilaria sp.	Red Weed

### Laminaria sp.	Kelp

### Potamogeton crispus	Curly Pondweed

### Ruppia maritima	Widgeon Grass

### Sargassum filipendula	Sargassum

### Ulva lactuca	Sea Lettuce

### Zostera marina	Eelgrass

### Brown filamentous algae

### Green filamentous algae

### Red-brown filamentous algae

3.  Palustrine Wetlands:

## Scientific Name	Common Name

Acer rubrum	Red Maple

Amelanchier sp.	Service-Berry

Amelanchier canadensis	Oblong-Leaf Service-Berry

Alnus rugosa	Speckled Alder

Cornus amomun	Silky Dogwood

Impatiens capensis	Spotted Touch–Me-Not

### Phragmites australis						Common Reed

Salix species	Willow species

Spiraea latifolia	Broad-Leaf Meadow Sweet

Smilax rotundifolia	Common Greenbrier

Typha angustifolia	Narrow-leaved Cattail

Triadenum virginicum	Marsh St. John’s-Wort

Toxicondendron radicans	Poison Ivy

Vaccinium corymbosum	Highbush Blueberry

Viburnum recognitum	Northern Arrow-Wood

## Appendix B

## Field Data Sheets